Code12

Access Controller Use Manual V2.0

1.Specifications:

1.1Technical parameters:

| Item | Value | Item | Value |
| --- | --- | --- | --- |
| Operating voltage | DC 12V±10% | Storage | 1000 |
| Operating current | < 100mA | Card type | EM(standard) /Mifare(optional) |
| Operating temperature | 0℃—60℃ | Reading distance | 1－5CM |
| Weight | 350g | Dimensions | 130*58*24mm |

1.2Factory Default:

| Item | Value | Item | Value |
| --- | --- | --- | --- |
| Authentication Mode | Card or Password(PIN) | Common PIN | None |
| Unlocking Duration | 5 S |  |  |

1.3Light and Sound indicates:

Light: red and green

| Light Description | Indicates |
| --- | --- |
| Red light flash every 2 sec(Slow flash) | Standby state |
| Green light flash every 0.5sec(Fast flash) | Pending further operation |
| Red light is always ON | Programming state |
| Green light is always ON | Unlocking state |
| Red and green light flash alternately | Restoring Factory Programming Password |

Sound: beep

| Sound Description | Indicates |
| --- | --- |
| 1 short beep | Valid input |
| 3 short beeps | Invalid input |
| 1 long beep | Programming state |
| 3 long beeps | Restoring Factory Programming Password |

2.Installations Guide:

Remark:

If let Relay drive electric-lock, please connect a backward diode

with the lock’s “+” and “-” poles.

3. Programming Guide

Programming Password Default: 123456, all password can be 3~6 digits

3.1 Detailed Programming Guide：

3.2  To reset to Factory Programming Password（123456）

Step1、Turn off power and connect bin 2&3 of RST(S1)。

Step2、Turn on power and buzzer gives 3 long buzz with green light flash fast。

If need to set Manager Card, now read two empty cards first one as Manager Add Card and Second one as Manager Delete Card with red light flashing and continuous short buzz .

Step3、Turn off power, disconnect bin2&3 and connect bin 1&2 of RST。

Step4、Turn on power again.

3.3  How to use Manager Card – MAC&MDC

To add a card user in standby mode:  read MAC  read new cards  read MAC

To delete a card user in standby mode: read MDC   read registered cards

read  MDC

To delete all users in standby mode:  read MDC  read MAC  read MDC

Remark: User cards can be added or deleted continuously.

3.4 User ID No. explain

User ID is auto-generating 4-digit number started at 0001. The number increases with adding user one by one and ignores deleting user.

Common Access Password has no ID.

3.5 How to change PIN (When open mode: card+ PIN)

After reading card and input corresponding PIN to open the door,

within 5 seconds, long press  #  until the green light flash fast， and

then input  new PIN   #   new PIN   # , new PIN changed successfully

with a long buzz.