[image: image1.png]Qualification Card




Dual frequency access control 
User Manual  V1.0
1. Specifications:                                        

1.1Technical parameters：                                                                               
	NO
	Item
	Value

	1
	Operating voltage
	AC/DC 12-24V

	2
	Operating current
	< 100mA

	3
	Ambient temperature
	-20℃—70℃

	4
	Relative humidity
	20%—80%

	5
	Storage
	10000 users

	6
	Card type (alternative)
	ID+IC dual band

	7
	Card reading distance
	1－8CM


1.2 Factory Default：       

[image: image2.png]NC LOCK
No_Lock |-
Blgck  Blue  Grey  Green  Brown  Pink
Red | Purple| Vellow Grey  White  Orange
4C CND PUSHOPEN BELLBELL D0 DI XC COM MO

ST

S| rWempit

Re\eise BELL |and output

IRelay output|





1.3 Light and Sound indicates:  
Light: red and green

	Light Description
	Access control status

	Red light flash every 1 sec （Slow flash）
	Holding state

	Red light is always ON 
	Programming state

	Green light is always ON within Open time
	Unlocking state

	Green light flash every 0.5sec (Fast flash)
	Pending further action


Sound：buzz

	Description
	Indicates

	1 short buzz
	Valid input

	3 short buzz
	Invalid input

	1 long buzz
	Program success

	Continuous long buzz
	Restoring factory programming passwords


2. Installations Guide:                                              
[image: image3.png]Programming ltem

Press key and swipe card operations

Remark

[Add user cara

[# Pro.pw # 1 swipe cards #

I add series cards, swipe cards continuously

|Configure Common PIN

[#Pro.PW# 21 new Common PIN #

|Common PINis only one, Unlocking method : Enter Common PIN then press [#]

Basic
Function Delete all users [# ProPw# 40 0000 # Delete all cards and PIN except Common PIN
Delete user by swiping card [# ProPW # 41 swipe cards # If delete series cards, swipe cards continuously
|Change Programming Password (Pro.PW) [# Pro.PW # 0 new Pro.PW # new Pro.PW # _|If Pro.PW lost, perform 3.2 to reset to Factory Default (123456)
[Add user by input User ID [ Pro.PW # 22 4-cigit User ID swipe card # |A00 USEr card or personal door opening password according to the number
|specified by the user. The user number must be 4 digits and unique. Personal
[Add PIN [# Pro.PW # 23 4-digit User ID new PIN# | password User + User ID A user can add a maximum of 1000 groups
Delete user by UserID [# ProPw # 42 4-digit User ID # [The user number is 4 digis, as describedin Section 3.3
Delete user by card No. [# ProPW# 43 caraNo.# |Cara No.is 10-digit or 8- digt, device auto-identiy
Expand |Configure Unlocking Duration [#ProPw# s XX bocis 2-digit (00~99). 1130 s 00, Unlocking Duration is 0.25
Funcion

|Configure Autnentication Mode

[#ProPwz 6 XX

o can be 01(only Caray02(Card or PIN)03(Card & PIN)

|Card Reader mode

[#ProPwz 6 XX

o can be 04WG26 Outputy05(WG34 Output)

|Automatic card adding mode

[#ProPw T X

X can be 05(Enter automatic card moge)06 (Exit automatic card mode)

Backiight setting

[#ProPw e XX

o can be 01102103 (Normal ON / Normal OFF / Auto)

Reset Factory Default

[#ProPwaBo0#

[Except Programming Password and Manager Card





Wiring diagram:



3.Programming Guide
3.1 Programming Guide: (Programming Password Default: 123456)



3.2 Initialize the programming password (123456) and add the management card

Step 1: Power off, short-connect the OPEN line and GND line, power on again, and the traffic light flashes alternately after three long sounds

Step 2: Swipe two blank cards in a row, the first one is "add card", the second one is "delete card" (if no management card is required, this step can be omitted)

Step 3: Power off, disconnect the connection between OPEN and GND, and then power on again, the programming password initialization is complete
3.3 Use ID explanation
User ID is 4-digit number from0001 to 9999. There is no User ID for the users added by
command [1]. Perform command [40] to clear all User ID.
4. Other extensions
4.1 How to change PIN
Swipe card or press PIN to unlock within 5s press [#] for3suntil the green light flash fast, then press [new PIN#new PIN#], the new PIN changed successfully with a long beep.
4.2 Authentication Mode for Card& PIN function
Setp1:When Authentication Mode is [Card or PIN] (factory default), according to the step 4.1 to configure the PIN of the card.
Setp2:Press [#Programming Password#603#] to configureAuthentication Mode to be [Card & PIN].
Setp3:Swipe user’s card with green light fast flash then input [PIN#]with green light normally ON, the door opened successfully.
4.3 Door open function description: 
When the credit card or password is normally
opened, press the 7, 9 digit key to keep the door
open. When the door is always open, only once
the credit card, password, or the door button is
opened again, the exit is always open.
Code49





Programming Password�
123456�
Common Access Password�
None�
�
Backlight�
Auto (for options)�
Relay Unlock output�
5 Seconds�
�
Open mode�
Card or Access Password


Access Password include Private Access Password (PIN) and Common Access Password�
�






NO�
Mark�
Color�
Function�
�
1�
AC�
Red�
AC/DC 12-24V�
�
2�
AC�
Black�
�
�
3�
GND�
Purple�
Power-�
�
4�
PUSH�
Blue�
Low level output�
�
5�
OPEN�
Yellow�
Door Release�
�
6�
BELL�
Grey�
BELL�
�
7�
BELL�
Grey�
BELL�
�
8�
D0�
Green�
WG DATA0�
�
9�
D1�
White�
WG DATA1�
�
10�
NC�
Brown�
Relay NC output�
�
11�
COM�
Orange�
Relay Com output�
�
12�
NO�
Pink�
Relay NO output�
�






Remark:


If let relay drive electric-lock, please connect a reverse diode with the lock’s “+” and “-” poles.





Remark: All password can be 3~6 digits








